CPE301 – SPRING 2019

# Design Assignment 2A

Student Name: Robert Sander

Student #: 5002102412

Student Email: sander1@unlv.nevada.edu

Primary Github address: https://github.com/sanderUNLV/submission_DA.git

Youtube link:

T1:  https://youtu.be/xdysO13sV3g

T2:  https://youtu.be/vJSh2bEIM3E

- CONNECTION BLOCK DIAGRAM w/ PINS

Block diagram with pins used in the Atmega328P

T1:

T2:

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

2.1.1.ASM

// DA2A_T1_A

// Author : Robert Sander

.include<m328pdef.inc>

.INCLUDE "DELAY_OFF_ON.asm"

.ORG 0x00

LDI R16, 0x3C	;LOAD THE VALUE 0x02 0000 0010

OUT DDRB, R16	;SETS PB.2-5 AS OUTPUTS

TOGGLE:

LDI R16, 0x38	;LOAD THE VALUE 0x38 INTO R16

OUT PORTB, R16	;'TURNS ON' PORTB HIGH STATE

RCALL DELAY_OFF	;CALL DELAY_OFF SUBROUTINE

LDI R16, 0x3C	;LOAD THE VALUE 0x3C INTO R16

OUT PORTB, R16	;'TURNS OFF' PORTB LOW STATE

RCALL DELAY_ON	;CALL DELAY_ON SUBROUTINE

RJMP TOGGLE		;GO TO 'TOGGLE:'

………………………

.ORG 0x000100

DELAY_OFF:

; Delay 4 640 000 cycles

; 290ms at 16.0 MHz

ldi  r18, 24

ldi  r19, 138

ldi  r20, 232

L2: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L2

ret              ; Return to the caller

DELAY_ON:

; Delay 6 960 000 cycles

; 435ms at 16.0 MHz

ldi  r18, 36

ldi  r19, 79

ldi  r20, 221

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

ret

2.1.1.C

// DA2A_T1_C

// Author : Robert Sander

#define F_CPU 16000000UL

#include <avr/io.h>     //include this input output library

#include <util/delay.h> //include this delay library

int main (void)

{

DDRB |= 0x3C;        //SETS PB.2-5 FOR OUTPUT

while (1) {

PORTB = ~(1<<2); //&= SET IT TO ZERO...TURNS THE LED ON...REVERSE LOGIC

_delay_ms(290);  //0.725 x 0.4=0.290s

PORTB = 0x3C;    // PUT 1'S IN ALL 8 BITS WHICH TURNS OFF THE LEDS 0011 1100 ONLY TURNS OFF PB.2-5

_delay_ms(435);  //0.725 x 0.6=0.435s - 60% DUTY CYCLE ON

}

return 0;

}

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2

3.1.1.ASM

// DA2A_T2_A

// Author : Robert Sander

.include <m328pdef.inc>

.include "DELAY.asm"

.ORG 0x000000

LDI R16, 0x3C	;LOAD THE VALUE 0x3C INTO R16

OUT DDRB, R16	;SETS PB.2-5 AS OUTPUTS, 00111100

CHECK:

SBIC PINC,2 ;skip next if PB2 is clear , IF BUTTON IS PUSHED GO TO "LDI R16, 0x38"

RJMP SKIP ;GO TO 'SKIP:'

LDI R16, 0x38 ;LOAD THE VALUE 0x38 INTO R16

OUT PORTB, R16	;'TURNS ON' PORTB...REVERSE LOGIC

RCALL DELAY

SKIP:

LDI R16, 0x3C ;LOAD THE VALUE 0x3C INTO R16

OUT PORTB, R16	;'TURNS OFF' PORTB...REVERSE LOGIC

RJMP CHECK ;GO TO 'CHECK:'

………………………

; Delay 20 000 000 cycles

; 1s 250ms at 16.0 MHz

.ORG 0x000100

DELAY:

ldi  r18, 102

ldi  r19, 118

ldi  r20, 194

L1: dec  r20

brne L1

dec  r19

brne L1

dec  r18

brne L1

RET

3.1.2.C

// DA2A_T2_C

// Author : Robert Sander

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

int main (void)

{

/* SET PORTB FOR OUTPUT*/

DDRB |= 0xFF;  //ALL BITS AS INPUTS TO TURN OFF THE BITS IN PINB

PORTB |= 0x3C; // TURN OFF LED PB.2-5

/* SET PORTC FOR INPUT*/     //PC.0 IS CONNECTED TO POTENTIOMETER

DDRC |= (1<<2);    //SETS PC.2 AS AN INPUT

PORTC |= (1 << 2); // ENABLES PULL-UP IN PC.2, PUTS SWITCH IN TO "OFF POSITION"

while (1) {

if (!(PINC & (1<<PINC2))) //CONSTANTLY POLL TO SEE CONDITION FOR PINC, CHECK TO SEE IF THE PIN IS HIGH, BUTTON IS PUSHED

{

PORTB &= ~(1<<2);   // SET PB.2 TO 0 TO TURN THE LED ON, REVERSE LOGIC

_delay_ms(1250);	//DELAY 1.25s

}

else

PORTB |= (1<<2);    // SET PB.2 TO 1 TO TURN OFF THE LED

}

return 0;

}

- SCHEMATICS

T1						T2

Figure 2 – Task 2 schematic

Figure 1 – Task 1 schematic

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

T1

Figure 3 – Duty Cycle and Period (Assembly)

Figure 4 – Duty Cycle and Period (C)

T2

Figure 5 – Duty Cycle, Period, and 1.250s delay (Assembly)

Figure 6 – Duty Cycle, Period, and 1.250s delay (C)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Figure 7 – Xplained Mini w/ Multifunction Shield and Logic Analyzer

- VIDEO LINKS OF EACH DEMO

T1:  https://youtu.be/xdysO13sV3g

T2:  https://youtu.be/vJSh2bEIM3E

- GITHUB LINK OF THIS DA

https://github.com/sanderUNLV/submission_DA.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

-Robert Sander