CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Robert Sander

Student #: 5002102412

Student Email: sander1@unlv.nevada.edu

Primary Github address: https://github.com/sanderUNLV/submission_DA.git

Youtube link: https://youtu.be/Ts_ygjckxOw

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

// DA3A_T.1_C

// Author : Robert Sander

#define F_CPU 16000000UL

#include <avr/io.h>

#include <stdio.h>

#include <avr/interrupt.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

//Declaration of our functions

void USART_init(void);

void USART_send(unsigned char data);

// String

void USART_putstring(char *StringPtr);

char String[] = "Hello! ";  //String[] is in fact an array but when we put the text between the " " symbols the compiler threats it as a String and automatically puts the null termination character in the end of the text

// Random Integer

void USART_putrandomint(char *randint);

volatile int randint = 8;

char randintout[2];

// Floating point number

void USART_putfloating(char *floatingnumber);

volatile double floatingnumber = 0.123;

char fltouts[20];

// Space

void USART_putspace(char *SpacePtr);

char Space[] = " ";

int main(void) {

OCR1A = 62500;			// 16000000/256=62500 - we set the new TOP number to 62500 for 1s delay

TCCR1B |= (1<<CS12);	//int_clk internal clk, 256 prescaler

TCCR1A |= (1<<WGM12);	//CTC mode TOP=OCR1A

TIMSK1 = (1<<OCIE1A);	//enable Timer1 compare match int.

sei ();			//enable interrupts

USART_init();        //Call the USART initialization code

while (1)

{

//Infinite loop

}

}

void USART_init(void) {

UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8);

UBRR0L = (uint8_t)(BAUD_PRESCALLER);

UCSR0B = (1 << TXEN0);

UCSR0C = (3 << UCSZ00);

}

void USART_send(unsigned char data) {

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = data;

}

void USART_putstring(char* StringPtr)

{

while (*StringPtr != 0x00) {

USART_send(*StringPtr);

StringPtr++;

}

}

void USART_putrandomint(char *randint)

{

while (*randint != 0x00) {

USART_send(*randint);

randint++;

}

}

void USART_putfloating(char *floatingnumber)

{

while (*floatingnumber != 0x00) {

USART_send(*floatingnumber);

floatingnumber++;

}

}

void USART_putspace(char* SpacePtr)

{

while (*SpacePtr != 0x00) {

USART_send(*SpacePtr);

SpacePtr++;

}

}

ISR (TIMER1_COMPA_vect)		//ISR for Timer0 compare match

{

USART_putstring(String);    //Pass the string to the USART_putstring function and sends it over the serial

snprintf(randintout,sizeof(randintout),"%d\r\n", randint);	// print random integer

USART_putrandomint(randintout);

USART_putstring(Space);

snprintf(fltouts,sizeof(fltouts),"%f\r\n", floatingnumber);  // print floating point number

USART_putfloating(fltouts);

TIFR0 |= (1<<OCF0A); // clear the compare A match flag

}

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Figure 1 – Data Visualizer Output, 1000ms delay

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/Ts_ygjckxOw

- GITHUB LINK OF THIS DA

https://github.com/sanderUNLV/submission_DA.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

-Robert Sander