CPE301 – SPRING 2019

# Mid 2/ Final

Student Name: Robert Sander

Student #: 5002102412

Student Email: sander1@unlv.nevada.edu

Primary Github address: https://github.com/sanderUNLV/submission_DA.git

Youtube link: https://youtu.be/9MAIOPtxEwQ

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

// Assignment: Midterm 2/ Final

// Author : Robert Sander

/*

* ATmega16 Interface with MPU-6050

* http://www.electronicwings.com

*

*/

#define F_CPU 16000000UL								/* Define CPU clock Frequency e.g. here its 8MHz */

#include <avr/io.h>										/* Include AVR std. library file */

#include <util/delay.h>									/* Include delay header file */

#include <inttypes.h>									/* Include integer type header file */

#include <stdlib.h>										/* Include standard library file */

#include <stdio.h>										/* Include standard library file */

#include "APDS9960_def.h"							/* Include MPU6050 register define file */

#include "I2C_Master_H_file.h"							/* Include I2C Master header file */

#include "USART_RS232_H_file.h"							/* Include USART header file */

#define APDS9960_WRITE 0x72

#define APDS9960_READ 0x73

float clear, red, blue, green;

void APDS9960_Init()										/* Gyro initialization function */

{

_delay_ms(150);										/* Power up time >100ms */

//I2C_Start_Wait(APDS9960_WRITE);								/* Start with device write address */

//I2C_Write(APDS9960_ENABLE);								/* Write to sample rate register */

//I2C_Write(0x00);									/* 1KHz sample rate */

//I2C_Stop();

//I2C_Start_Wait(APDS9960_WRITE);								/* Start with device write address */

//I2C_Write(APDS9960_ENABLE);								/* Write to sample rate register */

//I2C_Write(0x01);									/* 1KHz sample rate */

//I2C_Stop();

I2C_Start_Wait(APDS9960_WRITE);

I2C_Write(APDS9960_ATIME);								/* Write to power management register */

I2C_Write(DEFAULT_ATIME);									/* X axis gyroscope reference frequency */

I2C_Stop();

I2C_Start_Wait(APDS9960_WRITE);

I2C_Write(APDS9960_CONTROL);									/* Write to Configuration register */

I2C_Write(DEFAULT_AGAIN);									/* Fs = 8KHz */

I2C_Stop();

//I2C_Start_Wait(APDS9960_WRITE);								/* Start with device write address */

//I2C_Write(APDS9960_ENABLE);								/* Write to sample rate register */

//I2C_Write(1<<POWER|1<< AMBIENT_LIGHT);									/* 1KHz sample rate */

//I2C_Stop();

I2C_Start_Wait(APDS9960_WRITE);								/* Start with device write address */

I2C_Write(APDS9960_ENABLE);								/* Write to sample rate register */

I2C_Write(0x02);									/* 1KHz sample rate */

I2C_Stop();

}

void APDS_Start_Loc()

{

I2C_Start_Wait(APDS9960_WRITE);								/* I2C start with device write address */

I2C_Write(APDS9960_RDATAH);							/* Write start location address from where to read */

I2C_Repeated_Start(APDS9960_READ);							/* I2C start with device read address */

}

void Read_RawValue()

{

APDS_Start_Loc();									/* Read ambient light value */

clear = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

red = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

blue = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

green = (((int)I2C_Read_Ack()<<8) | (int)I2C_Read_Ack());

I2C_Stop();

}

int main()

{

char buffer[20], float_[10];

float C, R, G, B;

I2C_Init();											/* Initialize I2C */

APDS9960_Init();										/* Initialize MPU6050 */

USART_Init(9600);									/* Initialize USART with 9600 baud rate */

while(1)

{

USART_SendString("cool");

Read_RawValue();

C = clear;

R = red;

G = green;

B = blue;							/* Divide raw value by sensitivity scale factor to get real values */

dtostrf( C, 3, 2, float_ );					/* Take values in buffer to send all parameters over USART */

sprintf(buffer," CLEAR LIGHT = %s lumens ", float_);

USART_SendString(buffer);

dtostrf( R, 3, 2, float_ );					/* Take values in buffer to send all parameters over USART */

sprintf(buffer," RED LIGHT = %s lumens ", float_);

USART_SendString(buffer);

dtostrf( G, 3, 2, float_ );					/* Take values in buffer to send all parameters over USART */

sprintf(buffer," GREEN LIGHT = %s lumens ", float_);

USART_SendString(buffer);

dtostrf( B, 3, 2, float_ );					/* Take values in buffer to send all parameters over USART */

sprintf(buffer," BLUE LIGHT = %s lumens ", float_);

USART_SendString(buffer);

USART_SendString("beans!");

}

}

//USART_tx_string("AT+CWJAP=\"ssid\",\"password\"");

//USART_tx_string("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80");

//USART_tx_string("AT+CIPSEND=51");

//USART_tx_string("GET /update?key=KEY&field1=100");

//USART_tx_string("GET /update?key=key&field1=100");

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/9MAIOPtxEwQ

- GITHUB LINK OF THIS DA

https://github.com/sanderUNLV/submission_DA.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

-Robert Sander